Brian Lovegrove

4/26/25

90 Day Angelus 120/121L Seamer

Main Drive

- Checked main drive end play, found play when doing so.

- Removed the main cover.

- Loosened tension of the drive belt.

- Checked main drive pulley for vertical play and back lash, vertical tolerance is 0.003 to 0.005, back lash tolerance is 0.005 to 0.008. found vertical play to be at 0.020 and backlash to be at 0.31.

- Tightened main drive pully. To be noted when it is tightened there is 6.35mm of play before the seamer starts to engage. See attached video.

Turrets

- Drained oil from upper turret. Refilled with 4 quarts, to be noted upper turret had less than 4 quarts resting in it before I drained it, to be noted there was fine metal shavings/debris in the oil that was settled on the bottom.

- Drained oil from lower turret, refilled to center of sight glass 2 ½ quarts.

Lid / Cover Feed

- Visually inspected separator disc for wear, found it to be worn and have small nicks along with buildup of lid shavings.

- Rotated separator disc to check for backlash found it to be slightly over 1/8” and the tolerance is not to exceed 1/8” this could be the reason for cans randomly dropping out of the seamer without lids which has been called out by operators lately.

- Visually inspected knife found it to be worn, separator edge of knife is flat and needs to be replaced where it should have a radius on the cutting edge, this is likely the reason why lids continue to dispense for a couple pockets after the last can makes it through sometimes.

- Ensured knife is behind the cap / lid guide when retracted

- Checked the separator disc clearance, 0.018 between cover and separator, 0.014 between from finger to disc, tolerance between cover and separator is 0.010 to 0.015 and 0.003 to 0.010 between from finger to disc, made adjustments and was able to get it within tolerance on the high end at 0.0014 between cover and separator and 0.009 between from finger to disc, to be noted it felt like I would have to force the adjustment points if I went any further.

- Back guide to infeed finger is at 0.006 where tolerance is 0.003 to 0.005.

- Visually inspected all 12 cap pushers for grooves and tightness, found to be worn down, should be replaced.

- Checked operation of nc/nc solenoid, flagged sensor and hand cranked forward and observed knife movement, functioning properly.

- Checked operation of low cover sensor, moved lids away from sensor and confirmed prox is flagging properly.

- Checked the gassing turret for buildup in passage ways, found abundance of lid shavings, cleaned them out and hosed it off.

Lubrication Manzel: 120L

- Hand cranked to ensure pump is lubricating properly

- Checked hoses to verify the correct amount of drops, line 1 should be 6 to 10 and all others are 4 to 6, after inspecting found L1-8 L2-6 (L3-7 issue) (L4-9 issue) L5-4 L6-5 (L7-2/3 issue).

- Followed each hose coming out of the Manzel and inspected for breaks, cracks and leaks, found multiple leaks at the fittings, tightened and leaks were resolved.

- Verified flow at end points found all points to be dispensing oil properly.

Lubrication Oil Recirculation System: 121L

- Checked gauges all gauges and all have proper readings.

- Triggered low level on oil reservoir to ensure recirculation system shut off when triggered and threw fault.

- Checked for leaks, found no leaks, to be noted cabinet is soaked with oil most likely from refilling.

- Checked sight glass on bottom of seamer oil pan on seamer, oil was just slightly below mid-level.

Cover feed Rails

- Positioned #1 head at 90-degree mark.

- Visually inspected cap guide rails found them to be worn but still in good condition.

Feed turret

- Manually rotated feed turret from right to left and then left to right with my hands with dial indicator in place found play to be at 0.047 which is within tolerance 0.050

- Removed socket head cap screws that secure the feed turret upper hub to the feed turret, to be noted it did not drop when removed had to use a pry bar to drop it.

- Set up dial indicator with magnet base so that the dial indicator was positioned at bottom of feed turret. With my hands I rocked the feed turret up and down the indicator did not return to zero position. PM procedure states that the bearing should be replaced if it does not return to zero position or 0.030”

- Raised feed turret hub and shaft back up and installed socket head cap screws to secure it.

Cap Feed Timing

- Installed plug on Station 7

- Rotated plug backwards to makeup point where head #1 is between 88 and 89 degree mark and then moved it forward to the 90 degree mark

- Set proper clearance for inlet discharge rail to 0.002, tolerance is 0.000 to 0.002 verified with feeler gage.

- Checked clearance for finger chain found it to be at 0.210 with backlash out, adjusted to 0.120. tolerance is 0.096 to 0.125.

- Slid plug up at lid placement, fit smoothly without binding.

- Checked gap between plug and outside guide rail found it to be at 0.021 adjusted it to 0.09. tolerance is 0.010 plus or minus 0.003.

- Checked outside guide rail found it to be at 0.026 adjusted it to 0.020. tolerance is 0.020 plus or minus 0.003.

- Discharge star clearance was at 0.028, tolerance is 0.0220 to 0.0300 no adjustment needed.

Discharge Star

- Moved discharge star as far as it will go, placed dial indicator on outside edge of a pocket then rotated gear as far as it will go in the other direction, found play to be 1.53 whereas the tolerance is 0.0 to .140. extremely worn out along with the cup portion inside the star is full of metal shavings and grease although the star pockets are worn but do not have any burrs or anything that would damage cans. Needs to be rebuilt and gone through.

- Checked transfer onto discharge conveyor from dead plate, found it to be decently smooth could use a very thin shim on dead plate other than that it is in good condition.

Can Feed Chain

- Measured the distance of ten fingers sections from tip to tip, 3 different places on the chain measured out to, 37.8, 37.6, 37.5, tolerance is 36.5 to 38, chain is in spec and I believe was replaced recently.

- Visually inspected the entire chain looking for bent and worn fingers along with missing cotter pins and bad rollers on links. I did not find anything on the chain, and it is in good condition.

- Checked the chain tensioner to be sure it was in the center of travel, it was not in the center of the lane, the tensioner is not tensioning anything and does not function properly, needs to be replaced.

Banjo Box

- Removed connection to filler boot

- Used dial indicator at 1 inch of banjo box, measured radial play by raising the output shaft up and down, found 0.021 of play. Tolerance is a max of 0.005.

- Key stock on the output shaft 1 inch from banjo box has 0.006 of play, tolerance is 0.002 of play.

- Did not drain banjo box because I could not locate any FM 220 to refill it with along with, we just got a new one in stock and should be replaced during next down time, along with that I did crack and check the oil and it is milky.

Belt Drive

- Physically removed connection to filler and removed the drive belt

- With a dial indicator at 1 inch of belt drive box measured radial play by raising the output shaft up and down found it to have 0.008 of play, tolerance is 0.005.

- Checked belt for wear and tension, belt is worn and should be replaced.

Direct drive

- Used dial indicator at 1 inch on the output shaft and measured the radial play by moving the shaft up and down found it to have 0.006 of play, tolerance is 0.0 to 0.005.

- Clamped key stock on the output shaft 1 inch from the gearcase, measured back lash with dial indicator, found it to have 0.004 of play. Tolerance is 0.0 to 0.002 of play.

Spring Pressure

- Checked lower chuck spring pressure on all 12 stations, I looked in QC lab for 2nd ops test to verify that everything was in spec before I started this step but could not locate it. I’m assuming everything was in spec before we finished the run on Friday, bellow is a table for all results.

| Station | Spring Pressure |
| --- | --- |
| 1 | 106 lbs |
| 2 | 95 lbs |
| 3 | 105 lbs |
| 4 | 92 lbs |
| 5 | 97 lbs |
| 6 | 89 lbs (slightly low) |
| 7 | 93 lbs |
| 8 | 96 lbs |
| 9 | 90 lbs |
| 10 | 99 lbs |
| 11 | 91 lbs |
| 12 | 94 lbs |

- Brought each station around to the high point of 1st operation roll, put spring pressure gauge between chuck and the lower lifter plate, preloaded the indicator making sure I contacted the chuck, took it back to zero until indicator went to first mark, set .030 deflection and recorded spring pressure from gauge. See the table below for results.

| Station | Final Spring Pressure With .030 Deflection |
| --- | --- |
| 1 | 100 lbs |
| 2 | 107 lbs |
| 3 | 102 lbs |
| 4 | 91 lbs |
| 5 | 98 lbs |
| 6 | 90 lbs |
| 7 | 94 lbs |
| 8 | 97 lbs |
| 9 | 91 lbs |
| 10 | 101 lbs |
| 11 | 90 lbs |
| 12 | 96 lbs |

- After measuring each station all are in spec with .030 deflection, station 6 without deflection was slightly low. Station 3 had 12 lbs of difference telling me there could be an issue with the spring and or lower lifter parts (lifter rod, sleeve, guide bushing) other than that all stations seam to be in good condition.

Seamer Lever Setup

- Checked seam lever end play at the 80 degree mark, see table below for results.

| Station | Seam Lever End Play |
| --- | --- |
| 1 | 0.003” |
| 2 | 0.004” |
| 3 | 0.002” |
| 4 | 0.005” |
| 5 | 0.006” (slightly high) |
| 6 | 0.003” |
| 7 | 0.004” |
| 8 | 0.002” |
| 9 | 0.003” |
| 10 | 0.005” |
| 11 | 0.004” |
| 12 | 0.002” |

- placed dial indicator on roll and zeroed gauge, pried lever down to measure float, see table below for results.

| Station | Lever Float (Roll Movement) |
| --- | --- |
| 1 | 0.004” |
| 2 | 0.003” |
| 3 | 0.003” |
| 4 | 0.005” |
| 5 | 0.006” (slightly high) |
| 6 | 0.004” |
| 7 | 0.003” |
| 8 | 0.005” |
| 9 | 0.004” |
| 10 | 0.005” |
| 11 | 0.003” |
| 12 | 0.004” |

- seam lever end play and lever float for all stations were good except for station 5 where I got a slightly high measurement for both seam lever end play and lever float, seam lever end play tolerance is .002 to .005 and lever float tolerance is 0.003 to 0.005.

- checked all knockout pad bearings by rotating by hand, while none were in horrible shape, almost all had some slop to them and should be replaced sometime in the future.

- Checked for oil leaks at spindles, found oil sitting on the lifter pad on stations 3,5,7.